ITN 101

Reference Network

Golden Configuration Guide

Device-by-device IOS configurations for classroom deployment

Brightpoint Community College — BET Division — Spring 2026

Configuration Order Configure devices in this order for a clean first boot: 1. R1 (Core — establishes HSRP, DHCP, NTP master, NAT) 2. R2 (Core — joins HSRP, connects WAN to branch) 3. SW-DIST (Distribution — creates VLANs, trunks to both routers) 4. SW-ACC (Access — assigns port VLANs, uplinks to distribution) 5. R3 (Branch — connects WAN, establishes branch LAN + DHCP) Credentials: username admin / password cisco / enable secret cisco SSH on R1 and R2 ¦ Telnet on SW-DIST, SW-ACC, and R3

# Table of Contents

# Before You Begin

Ensure all devices are factory-defaulted before entering these configurations. If a device has an existing config, erase it first:

Router#  erase startup-config

Router#  reload

Switch#  delete flash:vlan.dat

Connect via console cable (9600 baud, 8N1). Enter each config section in order from top to bottom.

Physical Cabling Checklist R1 G0/0/1 → SW-DIST Gi1/0/1 (straight-through) R2 G0/0/0 → SW-DIST Gi1/0/2 (straight-through) R2 G0/0/1 → R3 G0/1 (crossover or straight-through with auto-MDIX) SW-DIST Gi1/0/3 → SW-ACC Fa0/24 (straight-through) SW-DIST Gi1/0/4 → WAP-1 (straight-through) R1 G0/0/0 → ISP/Campus network (if available) PCs/Servers → SW-ACC Fa0/1-12 as needed

# R1 — ISR 4221 (Core, HSRP Active VL10/40)

R1 is the primary core router. It handles the internet uplink (NAT), serves DHCP pools for VLANs 10, 20, and 40, acts as the NTP master clock, and runs HSRP as the active gateway for VLAN 10 (Students) and VLAN 40 (Wireless). It advertises a default route via OSPF so all routers can reach the internet.

Layer: Core | SSH | HSRP Priority 110 for VL10/40 | NAT + DHCP + NTP Master

! ============================================================ ! R1 - ISR 4221 ¦ CORE LAYER ! HSRP Active: VLAN 10, VLAN 40 ! HSRP Standby: VLAN 20 ! NAT, DHCP, NTP Master, OSPF, SSH ! ============================================================ enable configure terminal ! --- Hostname & Domain --- hostname R1 no ip domain-lookup ip domain-name itn101.lab ! --- Security --- enable secret cisco service password-encryption security passwords min-length 5 ! --- SSH (requires domain-name + RSA key) --- username admin privilege 15 secret cisco crypto key generate rsa modulus 2048 ! --- Console --- line console 0 password cisco login logging synchronous exec-timeout 30 0 exit ! --- VTY (SSH only) --- line vty 0 4 login local transport input ssh exec-timeout 30 0 exit ip ssh version 2 ! --- Banner --- banner motd # ***************************************************** * R1 - ITN 101 Lab ¦ Core Router (HSRP Active) * * Brightpoint Community College * * Authorized access only * ***************************************************** # ! ============================================================ ! INTERFACES ! ============================================================ ! --- G0/0/0: ISP Uplink (NAT Outside) --- ! NOTE: In the classroom, connect to campus network or leave ! disconnected. Use DHCP or assign a static IP from your ISP. interface GigabitEthernet0/0/0 description ISP_UPLINK ip address dhcp ip nat outside no shutdown exit ! --- G0/0/1: Trunk to SW-DIST (physical interface, no IP) --- interface GigabitEthernet0/0/1 description TRUNK_TO_SW-DIST no ip address no shutdown exit ! --- Subinterface: VLAN 10 - Students --- interface GigabitEthernet0/0/1.10 description VLAN10_STUDENTS encapsulation dot1Q 10 ip address 192.168.10.2 255.255.255.0 ip nat inside standby 10 ip 192.168.10.1 standby 10 priority 110 standby 10 preempt exit ! --- Subinterface: VLAN 20 - Faculty --- interface GigabitEthernet0/0/1.20 description VLAN20_FACULTY encapsulation dot1Q 20 ip address 192.168.20.3 255.255.255.0 ip nat inside standby 20 ip 192.168.20.1 standby 20 priority 100 standby 20 preempt exit ! --- Subinterface: VLAN 40 - Wireless --- interface GigabitEthernet0/0/1.40 description VLAN40_WIRELESS encapsulation dot1Q 40 ip address 192.168.40.2 255.255.255.0 ip nat inside standby 40 ip 192.168.40.1 standby 40 priority 110 standby 40 preempt exit ! --- Subinterface: VLAN 99 - Management --- interface GigabitEthernet0/0/1.99 description VLAN99_MANAGEMENT encapsulation dot1Q 99 ip address 172.16.0.1 255.255.255.0 exit ! ============================================================ ! DHCP POOLS ! ============================================================ ! --- Exclude gateway, HSRP, and static addresses --- ip dhcp excluded-address 192.168.10.1 192.168.10.10 ip dhcp excluded-address 192.168.20.1 192.168.20.10 ip dhcp excluded-address 192.168.40.1 192.168.40.10 ip dhcp pool VLAN10-STUDENTS network 192.168.10.0 255.255.255.0 default-router 192.168.10.1 dns-server 192.168.30.10 domain-name itn101.lab exit ip dhcp pool VLAN20-FACULTY network 192.168.20.0 255.255.255.0 default-router 192.168.20.1 dns-server 192.168.30.10 domain-name itn101.lab exit ip dhcp pool VLAN40-WIRELESS network 192.168.40.0 255.255.255.0 default-router 192.168.40.1 dns-server 192.168.30.10 domain-name itn101.lab exit ! ============================================================ ! NAT (PAT Overload) ! ============================================================ access-list 1 permit 192.168.10.0 0.0.0.255 access-list 1 permit 192.168.20.0 0.0.0.255 access-list 1 permit 192.168.30.0 0.0.0.255 access-list 1 permit 192.168.40.0 0.0.0.255 access-list 1 permit 172.16.0.0 0.0.0.255 ip nat inside source list 1 interface GigabitEthernet0/0/0 overload ! ============================================================ ! OSPF ! ============================================================ router ospf 1 router-id 1.1.1.1 network 192.168.10.0 0.0.0.255 area 0 network 192.168.20.0 0.0.0.255 area 0 network 192.168.40.0 0.0.0.255 area 0 network 172.16.0.0 0.0.0.255 area 0 passive-interface GigabitEthernet0/0/0 default-information originate exit ! ============================================================ ! NTP (Master Clock) ! ============================================================ ntp master 4 ! ============================================================ ! SAVE ! ============================================================ end copy running-config startup-config ! ============================================================ ! VERIFICATION COMMANDS ! ============================================================ ! show ip interface brief ! show standby brief ! show ip ospf neighbor ! show ip dhcp binding ! show ip nat translations ! show ntp status ! show ip ssh

# R2 — ISR 4221 (Core, HSRP Active VL20)

R2 is the second core router and HSRP partner. It is active for VLAN 20 (Faculty) and standby for VLANs 10 and 40. It also provides the WAN link to the branch router R3 via G0/0/1, configured with bandwidth 1544 to simulate a T1 leased line. R2 participates in OSPF but does not run NAT or DHCP.

Layer: Core | SSH | HSRP Priority 110 for VL20 | WAN Transit to Branch

! ============================================================ ! R2 - ISR 4221 ¦ CORE LAYER ! HSRP Active: VLAN 20 ! HSRP Standby: VLAN 10, VLAN 40 ! WAN link to R3 (Branch), OSPF, SSH ! ============================================================ enable configure terminal ! --- Hostname & Domain --- hostname R2 no ip domain-lookup ip domain-name itn101.lab ! --- Security --- enable secret cisco service password-encryption security passwords min-length 5 ! --- SSH --- username admin privilege 15 secret cisco crypto key generate rsa modulus 2048 ! --- Console --- line console 0 password cisco login logging synchronous exec-timeout 30 0 exit ! --- VTY (SSH only) --- line vty 0 4 login local transport input ssh exec-timeout 30 0 exit ip ssh version 2 ! --- Banner --- banner motd # ***************************************************** * R2 - ITN 101 Lab ¦ Core Router (HSRP Active 20) * * WAN Transit to Branch * * Authorized access only * ***************************************************** # ! ============================================================ ! INTERFACES ! ============================================================ ! --- G0/0/0: Trunk to SW-DIST (physical interface, no IP) --- interface GigabitEthernet0/0/0 description TRUNK_TO_SW-DIST no ip address no shutdown exit ! --- Subinterface: VLAN 10 - Students (Standby) --- interface GigabitEthernet0/0/0.10 description VLAN10_STUDENTS encapsulation dot1Q 10 ip address 192.168.10.3 255.255.255.0 standby 10 ip 192.168.10.1 standby 10 priority 100 standby 10 preempt exit ! --- Subinterface: VLAN 20 - Faculty (Active) --- interface GigabitEthernet0/0/0.20 description VLAN20_FACULTY encapsulation dot1Q 20 ip address 192.168.20.2 255.255.255.0 standby 20 ip 192.168.20.1 standby 20 priority 110 standby 20 preempt exit ! --- Subinterface: VLAN 40 - Wireless (Standby) --- interface GigabitEthernet0/0/0.40 description VLAN40_WIRELESS encapsulation dot1Q 40 ip address 192.168.40.3 255.255.255.0 standby 40 ip 192.168.40.1 standby 40 priority 100 standby 40 preempt exit ! --- Subinterface: VLAN 99 - Management --- interface GigabitEthernet0/0/0.99 description VLAN99_MANAGEMENT encapsulation dot1Q 99 ip address 172.16.0.2 255.255.255.0 exit ! --- G0/0/1: WAN to R3 (Simulated T1) --- interface GigabitEthernet0/0/1 description WAN_TO_R3_BRANCH ip address 10.0.1.1 255.255.255.252 bandwidth 1544 no shutdown exit ! ============================================================ ! OSPF ! ============================================================ router ospf 1 router-id 2.2.2.2 network 192.168.10.0 0.0.0.255 area 0 network 192.168.20.0 0.0.0.255 area 0 network 192.168.40.0 0.0.0.255 area 0 network 172.16.0.0 0.0.0.255 area 0 network 10.0.1.0 0.0.0.3 area 0 exit ! ============================================================ ! NTP (Client - syncs to R1) ! ============================================================ ntp server 172.16.0.1 ! ============================================================ ! SAVE ! ============================================================ end copy running-config startup-config ! ============================================================ ! VERIFICATION COMMANDS ! ============================================================ ! show ip interface brief ! show standby brief ! show ip ospf neighbor ! show ntp associations ! show ip ssh ! show ip route

# SW-DIST — Catalyst 1000 (Distribution)

The distribution switch is the L2 backbone of the network. It trunks to both core routers (R1 on Gi1/0/1, R2 on Gi1/0/2), the access switch (Gi1/0/3), and the wireless AP (Gi1/0/4). All trunk ports carry only the VLANs they need. SW-DIST is configured as the STP root bridge for all VLANs. Unused ports are assigned to VLAN 999 and shut down.

Layer: Distribution | Telnet | STP Root | L2 Trunk Hub

! ============================================================ ! SW-DIST - Catalyst 1000 ¦ DISTRIBUTION LAYER ! L2 Backbone: Trunks to R1, R2, SW-ACC, WAP-1 ! VLANs 10, 20, 30, 40, 99, 999 ! Telnet access ! ============================================================ enable configure terminal ! --- Hostname --- hostname SW-DIST no ip domain-lookup ! --- Security --- enable secret cisco service password-encryption ! --- Console --- line console 0 password cisco login logging synchronous exec-timeout 30 0 exit ! --- VTY (Telnet) --- line vty 0 4 password cisco login transport input telnet exec-timeout 30 0 exit line vty 5 15 password cisco login transport input telnet exit ! --- Banner --- banner motd # ***************************************************** * SW-DIST - ITN 101 Lab ¦ Distribution Switch * * Catalyst 1000 ¦ L2 Backbone * * Authorized access only * ***************************************************** # ! ============================================================ ! VLANs ! ============================================================ vlan 10 name STUDENTS exit vlan 20 name FACULTY exit vlan 30 name SERVERS exit vlan 40 name WIRELESS exit vlan 99 name MANAGEMENT exit vlan 999 name BLACKHOLE exit ! ============================================================ ! TRUNK PORTS ! ============================================================ ! --- Gi1/0/1: Trunk to R1 (Core) --- interface GigabitEthernet1/0/1 description TRUNK_TO_R1 switchport mode trunk switchport trunk allowed vlan 10,20,40,99 switchport trunk native vlan 99 no shutdown exit ! --- Gi1/0/2: Trunk to R2 (Core) --- interface GigabitEthernet1/0/2 description TRUNK_TO_R2 switchport mode trunk switchport trunk allowed vlan 10,20,40,99 switchport trunk native vlan 99 no shutdown exit ! --- Gi1/0/3: Trunk to SW-ACC (Access switch uplink) --- interface GigabitEthernet1/0/3 description TRUNK_TO_SW-ACC switchport mode trunk switchport trunk allowed vlan 10,20,30,99 switchport trunk native vlan 99 no shutdown exit ! --- Gi1/0/4: Trunk to WAP-1 (Wireless AP) --- interface GigabitEthernet1/0/4 description TRUNK_TO_WAP-1 switchport mode trunk switchport trunk allowed vlan 40,99 switchport trunk native vlan 99 no shutdown exit ! ============================================================ ! UNUSED PORTS --> BLACKHOLE ! ============================================================ interface range GigabitEthernet1/0/5 - 24 description UNUSED_BLACKHOLED switchport mode access switchport access vlan 999 shutdown exit ! ============================================================ ! MANAGEMENT SVI ! ============================================================ interface Vlan99 description MANAGEMENT_SVI ip address 172.16.0.11 255.255.255.0 no shutdown exit ip default-gateway 172.16.0.1 ! ============================================================ ! NTP (Client - syncs to R1) ! ============================================================ ntp server 172.16.0.1 ! ============================================================ ! SPANNING TREE (optional hardening) ! ============================================================ ! Make distribution switch the STP root for all VLANs spanning-tree vlan 10,20,30,40,99 root primary ! ============================================================ ! SAVE ! ============================================================ end copy running-config startup-config ! ============================================================ ! VERIFICATION COMMANDS ! ============================================================ ! show vlan brief ! show interfaces trunk ! show spanning-tree summary ! show ntp associations ! show ip interface brief

# SW-ACC — Catalyst 2960 (Access)

The access switch provides physical port connectivity for end devices. It assigns VLANs at the port level: Fa0/1-4 for students (VLAN 10), Fa0/5-8 for faculty (VLAN 20), and Fa0/9-12 for servers/branch devices (VLAN 30). Its trunk uplink to SW-DIST is on Fa0/24. PortFast is enabled on all access ports for faster convergence.

Layer: Access | Telnet | PortFast on access ports | Trunk uplink Fa0/24

! ============================================================ ! SW-ACC - Catalyst 2960 ¦ ACCESS LAYER ! End-device connectivity ! Trunk uplink to SW-DIST on Fa0/24 ! VLANs 10, 20, 30, 99, 999 ! Telnet access ! ============================================================ enable configure terminal ! --- Hostname --- hostname SW-ACC no ip domain-lookup ! --- Security --- enable secret cisco service password-encryption ! --- Console --- line console 0 password cisco login logging synchronous exec-timeout 30 0 exit ! --- VTY (Telnet) --- line vty 0 4 password cisco login transport input telnet exec-timeout 30 0 exit line vty 5 15 password cisco login transport input telnet exit ! --- Banner --- banner motd # ***************************************************** * SW-ACC - ITN 101 Lab ¦ Access Switch * * Catalyst 2960 ¦ End-Device Ports * * Authorized access only * ***************************************************** # ! ============================================================ ! VLANs ! ============================================================ vlan 10 name STUDENTS exit vlan 20 name FACULTY exit vlan 30 name SERVERS exit vlan 99 name MANAGEMENT exit vlan 999 name BLACKHOLE exit ! ============================================================ ! TRUNK UPLINK ! ============================================================ ! --- Fa0/24: Trunk to SW-DIST --- interface FastEthernet0/24 description TRUNK_TO_SW-DIST switchport mode trunk switchport trunk allowed vlan 10,20,30,99 switchport trunk native vlan 99 no shutdown exit ! ============================================================ ! ACCESS PORTS - VLAN 10 (Students) ! ============================================================ interface range FastEthernet0/1 - 4 description STUDENT_PC switchport mode access switchport access vlan 10 spanning-tree portfast no shutdown exit ! ============================================================ ! ACCESS PORTS - VLAN 20 (Faculty) ! ============================================================ interface range FastEthernet0/5 - 8 description FACULTY_PC switchport mode access switchport access vlan 20 spanning-tree portfast no shutdown exit ! ============================================================ ! ACCESS PORTS - VLAN 30 (Servers / Branch devices) ! ============================================================ interface range FastEthernet0/9 - 12 description SERVER_PORT switchport mode access switchport access vlan 30 spanning-tree portfast no shutdown exit ! ============================================================ ! UNUSED PORTS --> BLACKHOLE ! ============================================================ interface range FastEthernet0/13 - 23 description UNUSED_BLACKHOLED switchport mode access switchport access vlan 999 shutdown exit ! ============================================================ ! MANAGEMENT SVI ! ============================================================ interface Vlan99 description MANAGEMENT_SVI ip address 172.16.0.12 255.255.255.0 no shutdown exit ip default-gateway 172.16.0.1 ! ============================================================ ! NTP (Client - syncs to R1) ! ============================================================ ntp server 172.16.0.1 ! ============================================================ ! SAVE ! ============================================================ end copy running-config startup-config ! ============================================================ ! VERIFICATION COMMANDS ! ============================================================ ! show vlan brief ! show interfaces trunk ! show interfaces status ! show spanning-tree summary ! show ntp associations ! show ip interface brief

# R3 — ISR 1941 (Branch Router)

R3 represents a remote branch office connected to the campus via a simulated T1 link (G0/1 to R2). It provides the default gateway for VLAN 30 (192.168.30.1) and runs a local DHCP pool for the branch LAN. The HWIC-2T serial module provides S0/0/0 and S0/0/1 for dedicated serial WAN lab exercises. R3 uses Telnet rather than SSH, providing a deliberate contrast with the 4221 routers.

Layer: Branch | Telnet | DHCP for VLAN 30 | Serial ports for lab exercises

! ============================================================ ! R3 - ISR 1941 ¦ BRANCH ROUTER ! WAN to R2 via GigabitEthernet ! Branch LAN: VLAN 30 (192.168.30.0/24) ! DHCP for branch, OSPF, NTP Client ! Telnet access (contrast with SSH on 4221s) ! Serial ports (S0/0/0, S0/0/1) available for lab exercises ! ============================================================ enable configure terminal ! --- Hostname --- hostname R3 no ip domain-lookup ! --- Security --- enable secret cisco service password-encryption ! --- Console --- line console 0 password cisco login logging synchronous exec-timeout 30 0 exit ! --- VTY (Telnet -- note: less secure than SSH on R1/R2) --- line vty 0 4 password cisco login transport input telnet exec-timeout 30 0 exit ! --- Banner --- banner motd # ***************************************************** * R3 - ITN 101 Lab ¦ Branch Router * * ISR 1941 ¦ Serial Labs Available * * Authorized access only * ***************************************************** # ! ============================================================ ! INTERFACES ! ============================================================ ! --- G0/1: WAN to R2 (Simulated T1) --- interface GigabitEthernet0/1 description WAN_TO_R2_CAMPUS ip address 10.0.1.2 255.255.255.252 bandwidth 1544 no shutdown exit ! --- G0/0: Branch LAN --- interface GigabitEthernet0/0 description BRANCH_LAN_VLAN30 ip address 192.168.30.1 255.255.255.0 no shutdown exit ! --- Serial Interfaces (available for lab exercises) --- ! NOTE: These are on the HWIC-2T module in EHWIC slot 0. ! Leave shutdown until needed for serial-specific labs. ! When used: clock rate 64000 on DCE end, bandwidth 64 interface Serial0/0/0 description SERIAL_LAB_PORT_0 no ip address shutdown exit interface Serial0/0/1 description SERIAL_LAB_PORT_1 no ip address shutdown exit ! ============================================================ ! DHCP POOL (Branch LAN) ! ============================================================ ! --- Exclude gateway and static server addresses --- ip dhcp excluded-address 192.168.30.1 192.168.30.20 ip dhcp pool VLAN30-BRANCH network 192.168.30.0 255.255.255.0 default-router 192.168.30.1 dns-server 192.168.30.10 domain-name itn101.lab exit ! ============================================================ ! OSPF ! ============================================================ router ospf 1 router-id 3.3.3.3 network 10.0.1.0 0.0.0.3 area 0 network 192.168.30.0 0.0.0.255 area 0 passive-interface GigabitEthernet0/0 exit ! ============================================================ ! NTP (Client - syncs to R1 via OSPF-learned route) ! ============================================================ ntp server 172.16.0.1 ! ============================================================ ! SAVE ! ============================================================ end copy running-config startup-config ! ============================================================ ! VERIFICATION COMMANDS ! ============================================================ ! show ip interface brief ! show ip ospf neighbor ! show ip route ! show ip dhcp binding ! show ntp associations ! show controllers serial 0/0/0 ! (check if DCE or DTE -- DCE needs clock rate)

# Post-Configuration Verification

After all five devices are configured and cabled, run these checks to confirm everything is working:

## HSRP Verification (on R1 or R2)

R1# show standby brief

Confirm R1 shows Active for groups 10 and 40, Standby for group 20. R2 should be the reverse.

## OSPF Verification

R1# show ip ospf neighbor

R1 should show R2 as a neighbor. R2 should show both R1 and R3.

R3# show ip route ospf

R3 should have OSPF-learned routes to 192.168.10.0, .20.0, .40.0, and 172.16.0.0, plus a default route (O*E2).

## DHCP Verification

R1# show ip dhcp binding

Connect a PC to SW-ACC Fa0/1 (VLAN 10). It should receive an address in 192.168.10.x range with gateway .1.

## End-to-End Connectivity

From a VLAN 10 PC, test each layer of the troubleshooting sequence:

PC> ping 192.168.10.1       ! HSRP gateway

PC> ping 192.168.20.1       ! Inter-VLAN routing

PC> ping 192.168.30.1       ! WAN to branch

PC> ping 10.0.1.1           ! WAN link

HSRP Failover Test To demonstrate HSRP failover to students: 1. From a VLAN 10 PC, start a continuous ping: ping -t 192.168.10.1 2. On R1, shutdown the subinterface: interface G0/0/1.10 / shutdown 3. Observe: 1-3 pings drop, then traffic resumes through R2 4. On R1, re-enable: interface G0/0/1.10 / no shutdown 5. With preemption, R1 reclaims Active status within seconds This is a compelling live demo of why HSRP exists.