ITN 101

Introduction to Network Concepts

Reference Network Documentation

3-Tier Architecture with HSRP Redundancy

Brightpoint Community College

BET Division — Midlothian Campus

Spring 2026

About This Document This document describes the reference network used throughout ITN 101. Every lab, demonstration, and exam scenario maps back to this topology. The network follows a 3-tier enterprise model (Core, Distribution, Access) with HSRP redundancy at the core and a simulated WAN branch. Keep this document accessible — you will refer to it all semester.

# Table of Contents

# Network Architecture Overview

The ITN 101 reference network follows a three-tier hierarchical design model — the same architecture used in enterprise campus networks. This model separates the network into Core, Distribution, and Access layers, each with distinct responsibilities.

## The Three Tiers

Core Layer (ISR 4221 x2): The two 4221 routers form an HSRP pair providing redundant default gateways. R1 handles the internet uplink and serves as the primary gateway for VLANs 10 and 40. R2 serves as the primary gateway for VLAN 20 and provides the WAN link to the branch. If either router fails, the other takes over all gateway duties automatically.

Distribution Layer (Catalyst 1000): The C1000 switch acts as the L2 backbone connecting all network segments. It trunks to both core routers, the access switch, and the wireless AP. All inter-VLAN traffic passes through this switch on its way to a core router for routing decisions.

Access Layer (Catalyst 2960): The C2960 provides physical port access for end devices. It enforces VLAN membership at the port level and connects upstream to the distribution switch via trunk.

Branch (ISR 1941): A remote-site router connected to R2 via a GigabitEthernet link configured to simulate a T1 leased line. The 1941 also has a HWIC-2T serial module with two serial interfaces available for dedicated serial WAN lab exercises.

Why 3-Tier + HSRP? The 3-tier model separates concerns: core handles routing and redundancy, distribution aggregates and enforces policy, access connects endpoints. HSRP ensures no single router failure takes down the network. Load-balancing HSRP means both routers carry traffic — no idle standby. This maps directly to CompTIA Network+ and CCNA exam objectives.

# Lab Rack Topology (Physical)

The following diagram shows the Brightpoint lab rack. From top to bottom: CyberPower CPS-1215RM PDU, two Cisco ISR 4221 routers (core), one Cisco ISR 1941 (branch), Catalyst 1000 (distribution), and Catalyst 2960 (access).

[Internet / ISP]

|

G0/0/0 (ISP-assigned)

+--------------------+

| R1 - ISR 4221      |  CORE (HSRP Active VL10/40)

| HSRP Pri: 110/100  |

+--------------------+

G0/0/1 |

(trunk)     |

+---+-------------------+-------------------+---+

|        SW-DIST — Catalyst 1000                |

|     DISTRIBUTION LAYER (L2 backbone)          |

|   VLANs: 10, 20, 40, 99 (trunked)            |

+--+--------+--------+--------+--------+-------+

|        |        |        |        |

Gi1/0/1  Gi1/0/2  Gi1/0/3  Gi1/0/4  Gi1/0/5

(R1)     (R2)     (SW-ACC)  (AP)     (spare)

|                   |

G0/0/0                [WAP-1]

+--------------------+    Wireless AP

| R2 - ISR 4221      |   VLAN 40

| HSRP Pri: 100/110  |   192.168.40.0/24

+--------------------+

G0/0/1 |

10.0.1.1  | WAN Link (Simulated T1)

| 10.0.1.0/30

10.0.1.2  |

G0/1 |

+--------------------+

| R3 - ISR 1941      |  BRANCH ROUTER

| Serial: S0/0, S0/1 |  (serial labs)

+--------------------+

G0/0 |

192.168.30.1

|

+-------+-------------+

|  SW-ACC — Cat 2960  |

|    ACCESS LAYER     |

+--+------+------+---+

|      |      |

Fa0/1  Fa0/2  Fa0/3

|      |      |       Fa0/24 → SW-DIST trunk

[PC-A]  [PC-B] [SRV-1]

VL10    VL20   VL30

DNS/DHCP

## Device Inventory

| Label | Model | Role / Layer | Mgmt IP | IOS Type |
| --- | --- | --- | --- | --- |
| R1 | ISR 4221 | Core (HSRP Active 10/40) | 172.16.0.1 | IOS XE 16.x |
| R2 | ISR 4221 | Core (HSRP Active 20) | 172.16.0.2 | IOS XE 16.x |
| R3 | ISR 1941 | Branch Router | 172.16.0.3 | IOS 15.x |
| SW-DIST | Catalyst 1000 | Distribution (L2) | 172.16.0.11 | IOS |
| SW-ACC | Catalyst 2960 | Access | 172.16.0.12 | IOS 15.x |
| WAP-1 | (TBD AP) | Wireless Access | 172.16.0.20 | N/A |

# HSRP Redundancy Design

Hot Standby Router Protocol (HSRP) provides a virtual IP address that acts as the default gateway for each VLAN. Two physical routers share the virtual IP. If the active router fails, the standby takes over within seconds.

This network uses HSRP load balancing: R1 is active for VLANs 10 and 40, while R2 is active for VLAN 20. This means both routers carry traffic during normal operation — neither sits idle.

| VLAN | Virtual IP (Gateway) | R1 (Physical IP / Priority) | R2 (Physical IP / Priority) |
| --- | --- | --- | --- |
| 10 (Students) | 192.168.10.1 | 192.168.10.2 / 110 (Active) | 192.168.10.3 / 100 (Standby) |
| 20 (Faculty) | 192.168.20.1 | 192.168.20.3 / 100 (Standby) | 192.168.20.2 / 110 (Active) |
| 40 (Wireless) | 192.168.40.1 | 192.168.40.2 / 110 (Active) | 192.168.40.3 / 100 (Standby) |

HSRP Key Concepts The virtual IP (.1) is what every host uses as its default gateway. Physical IPs (.2 and .3) are used for router-to-router HSRP communication. Higher priority = active router. Default priority is 100. If the active router fails, the standby promotes itself within ~3 seconds. Preemption: when the original active recovers, it reclaims the active role. HSRP group number typically matches the VLAN ID for easy reference.

# IP Addressing Scheme

LAN subnets use 192.168.x.0/24 where the third octet matches the VLAN ID. WAN links use 10.0.x.0/30. Management uses 172.16.0.0/24.

## Full Interface Addressing

| Device | Interface | IP Address | Description | Layer | HSRP Role |
| --- | --- | --- | --- | --- | --- |
| R1 | G0/0/0 | ISP-assigned | Internet uplink | Core | — |
| R1 | G0/0/1.10 | 192.168.10.2/24 | VLAN 10 sub-if | Core | Active (110) |
| R1 | G0/0/1.20 | 192.168.20.3/24 | VLAN 20 sub-if | Core | Standby (100) |
| R1 | G0/0/1.40 | 192.168.40.2/24 | VLAN 40 sub-if | Core | Active (110) |
| R2 | G0/0/0.10 | 192.168.10.3/24 | VLAN 10 sub-if | Core | Standby (100) |
| R2 | G0/0/0.20 | 192.168.20.2/24 | VLAN 20 sub-if | Core | Active (110) |
| R2 | G0/0/0.40 | 192.168.40.3/24 | VLAN 40 sub-if | Core | Standby (100) |
| R2 | G0/0/1 | 10.0.1.1/30 | WAN to R3 (sim T1) | Core | — |
| R3 | G0/1 | 10.0.1.2/30 | WAN to R2 (sim T1) | Branch | — |
| R3 | G0/0 | 192.168.30.1/24 | Branch LAN gateway | Branch | — |
| R3 | S0/0/0, S0/0/1 | (Lab use) | HWIC-2T serial ports | Branch | — |
| SW-DIST | VLAN 99 SVI | 172.16.0.11/24 | Management | Dist | — |
| SW-ACC | VLAN 99 SVI | 172.16.0.12/24 | Management | Access | — |
| WAP-1 | Mgmt | 172.16.0.20/24 | Wireless AP | Access | — |

# VLAN Architecture

VLANs segment the network at Layer 2. Each VLAN maps to its own IP subnet. Traffic between VLANs must pass through a core router (inter-VLAN routing via router-on-a-stick on R1 and R2). The distribution switch (C1000) trunks all VLANs to both core routers for HSRP to function.

| ID | Name | Subnet | Gateway | Purpose |
| --- | --- | --- | --- | --- |
| 10 | STUDENTS | 192.168.10.0/24 | 192.168.10.1 (HSRP) | Wired student workstations |
| 20 | FACULTY | 192.168.20.0/24 | 192.168.20.1 (HSRP) | Faculty/instructor devices |
| 30 | SERVERS | 192.168.30.0/24 | 192.168.30.1 (R3) | Branch: DNS, DHCP, file svcs |
| 40 | WIRELESS | 192.168.40.0/24 | 192.168.40.1 (HSRP) | Wireless AP clients |
| 99 | MGMT | 172.16.0.0/24 | 172.16.0.1 (R1) | Switch/router management |
| 999 | BLACKHOLE | N/A | N/A | Unused ports (security) |

## Switch Port Assignments

### SW-DIST — Catalyst 1000 (Distribution)

Central L2 backbone. All uplinks are 802.1Q trunks carrying VLANs 10, 20, 40, and 99.

| Port | Mode | VLANs | Connected To |
| --- | --- | --- | --- |
| Gi1/0/1 | Trunk | 10,20,40,99 | R1 G0/0/1 (Core — router-on-a-stick) |
| Gi1/0/2 | Trunk | 10,20,40,99 | R2 G0/0/0 (Core — router-on-a-stick) |
| Gi1/0/3 | Trunk | 10,20,30,99 | SW-ACC Fa0/24 (Access switch uplink) |
| Gi1/0/4 | Trunk | 40,99 | WAP-1 (Wireless AP) |
| Gi1/0/5-24 | Access | 999 | Unused (blackholed) |

### SW-ACC — Catalyst 2960 (Access)

Provides end-device connectivity. Uplinks to SW-DIST via trunk on Fa0/24.

| Port | Mode | VLAN | Connected To |
| --- | --- | --- | --- |
| Fa0/24 | Trunk | 10,20,30,99 | SW-DIST Gi1/0/3 (distribution uplink) |
| Fa0/1-4 | Access | 10 | Student workstations (PC-A, PC-B, etc.) |
| Fa0/5-8 | Access | 20 | Faculty devices (PC-C, etc.) |
| Fa0/9-12 | Access | 30 | Branch servers (SRV-1 DNS/DHCP, SRV-2 File) |
| Fa0/13-23 | Access | 999 | Unused (blackholed) |

# Key Protocol Roles

Every protocol has a home. Knowing which device handles which role is the foundation of troubleshooting.

| Protocol | Device | Details |
| --- | --- | --- |
| HSRP | R1 + R2 | Virtual gateways for VLANs 10, 20, 40. Load-balanced: R1 active for 10/40, R2 active for 20. |
| DHCP | R1 + SRV-1 | R1 serves pools for VLANs 10, 20, 40 (.100-.199). SRV-1 serves VLAN 30 (.100-.199). |
| DNS | SRV-1 | Primary DNS at 192.168.30.10. All DHCP pools point here. |
| Default GW | HSRP VIPs / R3 | VLANs 10/20/40: HSRP virtual .1 in each subnet. VLAN 30: R3 at 192.168.30.1. |
| NAT (PAT) | R1 | Overload on G0/0/0 for internet access from all internal subnets. |
| Routing | R1, R2, R3 | OSPF Area 0 across all routers. All subnets advertised. R2 is transit between campus and branch. |
| NTP | R1 (master) | NTP master stratum 4. R2, R3, SW-DIST, SW-ACC sync to R1. |
| Syslog | SRV-1 | Centralized logging at 192.168.30.10, port 514/UDP. |

Troubleshooting Sequence When a host cannot reach the internet, check in this order: 1. Can the host ping its own IP? (NIC/TCP-IP stack) 2. Can it ping the HSRP virtual gateway (.1)? (L2/VLAN/HSRP) 3. Can it ping the DNS server (192.168.30.10)? (inter-VLAN routing/WAN) 4. Can it ping a public IP like 8.8.8.8? (NAT/internet) 5. Can it resolve a domain name? (DNS) Each step isolates a different layer of the problem.

# Simplified Reference Network

This is the conceptual version, stripped of hardware-specific details. Use this model for subnetting exercises, routing table problems, or exam questions with generic device names.

[Cloud / Internet]

|

+===========+===========+  CORE LAYER

|  Router A   Router B  |  (HSRP pair)

|  Active     Standby   |  Virtual GW: .1

+===========+===========+

|

+------+------+       DISTRIBUTION

| Dist Switch |       (L2 backbone)

+--+--+--+--+-+

|  |  |  |

/   |  |   \____[Wireless AP]

/    |  |         VL40

[Branch]   |  |

Router C   |  |         ACCESS LAYER

|    +---+--+---+

|    | Acc Switch|

|    +--+--+--+-+

|       |  |  |

[LAN C] [PCs][Servers]

## Quick Reference Table

| Network | Subnet | Key Addresses |
| --- | --- | --- |
| Students (VL10) | 192.168.10.0/24 | GW (HSRP): .1 ¦ R1: .2 ¦ R2: .3 ¦ DHCP: .100-.199 |
| Faculty (VL20) | 192.168.20.0/24 | GW (HSRP): .1 ¦ R2: .2 ¦ R1: .3 ¦ DHCP: .100-.199 |
| Servers (VL30) | 192.168.30.0/24 | GW: .1 (R3) ¦ DNS: .10 ¦ DHCP: .100-.199 |
| Wireless (VL40) | 192.168.40.0/24 | GW (HSRP): .1 ¦ R1: .2 ¦ R2: .3 ¦ DHCP: .100-.199 |
| WAN (R2-R3) | 10.0.1.0/30 | R2: .1 ¦ R3: .2 (simulated T1) |
| Management | 172.16.0.0/24 | R1: .1 ¦ R2: .2 ¦ R3: .3 ¦ SW-D: .11 ¦ SW-A: .12 ¦ AP: .20 |

Patterns to Memorize Third octet = VLAN ID: VL10 → 192.168.10.x, VL20 → .20.x, VL30 → .30.x, VL40 → .40.x HSRP virtual gateway is always .1 — this is what hosts use. Active router gets .2, standby gets .3 (within each VLAN subnet). WAN links use /30 masks (only 2 usable hosts per link). Management network (172.16.0.0/24) is out-of-band from user traffic. HSRP is load-balanced: R1 active for VL10/40, R2 active for VL20.